Zone Voice Alarm Remote Microphone

EN-6500TM

Features:

- Digital voice evacuation system remote paging microphone.

- Each microphone of 120 zone capacity.

- Support 32 remote microphones of loop connection.

- Remote paging microphone with touch screen display.

- System indicators of AC, DC, fault, Mic status and test.

- 120 zone selection buttons through touch screen and all zone/talk button.

- Two RJ45 ports for connection with EVAC-500 and next remote microphone.

- One 3.5mm line input, with output volume control.

- Support PPT and normal paging two modes.

- DC24V power supply or POE.

- Support loop connection and paging microphone failure has been supervised.

- With free software to monitor the microphone working status and fault.

Specifications:

| Model | EN-6500TM |
| --- | --- |
| Description | Voice Alarm Remote Microphone (Touch Screen) |
| Power Consumption | 3W |
| Zone Capacity | 120 Zones |
| System Capacity | 32 Remote microphones |
| Line Inputs | 350mV, 10kΩ |
| Mic Input | 5mV, 600Ω |
| Frequency Response | 80Hz~16kHz |
| THD | ＜1% at RMS,1KHz |
| S/N Ratio | >70dB |
| Operating Temp | -20℃ to +70℃ |
| Power Supply | DC24V, ±20% |
| Humidity | <95% no condensation |
| Dimension | 256(W)×149(H)×52(D) mm |
| Weight | 1.5kg |

Rear Panel: