| Camera | Image Sensor | 1/2.8” Global Shutter CMOS |  |  |
| --- | --- | --- | --- | --- |
|  | Minimum Illumination | Color:0.001 Lux @ (F1.67, AGC ON); B/W:0.0005Lux @ (F1.67, AGC ON) |  |  |
|  | Shutter | 1/25s to 1/100,000s; Support delayed shutter |  |  |
|  | Day/Night Switch | Auto ICR cut filter |  |  |
| Lens | Focal Length | 5.4~162mm, 30x Optical Zoom |  |  |
|  | Aperture Range | F1.67-F3.67 |  |  |
|  | Field of View | H:43.84-1.53° (wide-tele) V:25.11-0.86° (wide-tele) D:49.82-1.75° (wide-tele) |  |  |
|  | Minimum Working Distance | 100mm-1500mm (wide-tele) |  |  |
|  | Zoom Speed | Approximately 4s(optical, wide-tele) |  |  |
| Compression Standard | Video Compression | H.265 / H.264 |  |  |
|  | H.265 Type | Main Profile |  |  |
|  | H.264 Type | BaseLine Profile / Main Profile / High Profile |  |  |
|  | Video Bitrate | 32 Kbps~16Mbps |  |  |
|  | Audio Compression | G.711a/G.711u/G.722.1/G.726/MP2L2/AAC/PCM |  |  |
|  | Audio Bitrate | 64Kbps(G.711)/16Kbps(G.722.1)/16Kbps(G.726)/32-192Kbps(MP2L2)/16-64Kbps(AAC) |  |  |
| Image | Main Stream | 50Hz: 50fps (1920x1080,1280x960,1280x720) 60Hz: 60fps (1920x1080,1280x960,1280x720) |  |  |
|  | Sub Stream | 50Hz: 50fps (1280x720,704x576, 640x480,352x288) 60Hz: 60fps (1280x720,704x480, 640x480,352x240) |  |  |
|  | Third Stream | 50Hz: 50fps (704x567,640x480,352x288) 60Hz: 60fps (704x480,640x480,352x240) |  |  |
|  | Image Settings | Saturation, Brightness, Contrast and Sharpness can be adjusted via the client-side or browse |  |  |
|  | BLC | Support |  |  |
|  | Exposure Mode | AE / Aperture Priority / Shutter Priority / Manual Exposure |  |  |
|  | Focus Mode | Auto Focus / One Focus / Manual Focus / Semi-Auto Focus |  |  |
|  | Area Exposure / Focus | Support |  |  |
|  | Defog | Support |  |  |
|  | Stability | EIS |  |  |
|  | Day/Night Switch | Automatic, manual, timing, alarm trigger |  |  |
|  | 3D Noise Reduction | Support |  |  |
|  | Picture Overlay Switch | Support BMP 24-bit image overlay, customizable area |  |  |
|  | Region of Interest | Support three streams and four fixed areas |  |  |
| Network | Storage Function | Support micro SD / SDHC / SDXC card (256g) offline local storage, NAS (NFS, SMB / CIFS support) |  |  |
|  | Protocols | TCP/IP,ICMP,HTTP,HTTPS,FTP,DHCP,DNS,RTP,RTSP,RTCP,NTP,SMTP,SNMP,IPv6 |  |  |
|  | Interface Protocol | ONVIF(PROFILE S,PROFILE G) |  |  |
| Intelligent | Basic Intelligence | Motion detection, video blocking, disk detection, illegal access |  |  |
|  | Smar Intelligent | Area intrusion, cross-border detection, leaving the area, wandering detection, crowd gathering, fast movement, audio anomaly |  |  |
| Interface | External Interface | 36pin FFC (Network port, RS485, RS232, SDHC, Alarm In/Out Line In/Out, power) USB3.0 |  |  |
|  | Digital Interface | N/A | LVDS | MIPI |
| General | Working Temperature | -30℃~60℃, humidity≤95%(non-condensing) |  |  |
|  | Power supply | DC12V±10% |  |  |
|  | Power consumption | 2.8W MAX (ICR, 4.2W MAX) |  |  |
|  | Dimensions | 95.3*50*61.5mm |  |  |
|  | Weight | 257g |  |  |