35x camera core module – Parameters

Features

| Category | Item | Description |
| --- | --- | --- |
| General Parameters | Main Chip | Hi3516DV500, ARM Cortex A55@850MHz dual-core |
|  | DDR (Memory) | 8 Gbit LP DDR4 |
|  | Flash | 1 Gb (2 Gb optional for bulk orders) |
|  | Image Sensor | 4MP 1/1.8" IMX664 low-illumination CMOS sensor Color: 0.0001 Lux@F1.0 B/W: 0.0001 Lux@F1.0 |
|  | Watchdog | Hardware watchdog |
|  | WDR | 120 dB |
| Image Parameters | Main Stream | 30 fps@ (2688×1520, 2560×1440, 1920×1080) |
|  | Sub Stream | 30 fps@ (704×576, 640×480) |
|  | Third Stream | 30 fps@ (704×576, 640×480) |
|  | Video Compression | H.265+ / H.265 / H.264 |
|  | Bitrate | 512 Kbps ~ 32 Mbps |
|  | White Balance | Auto / Manual / One-Push |
|  | Shutter | 1/12 s to 1/100,000 s |
|  | Noise Reduction | 3D / 2D noise reduction |
|  | WDR (Digital) | Digital WDR supported |
|  | Auto Iris | DC IRIS auto iris supported |
|  | AISP | AISP image enhancement supported |
|  | Image Functions | Backlight compensation, highlight suppression, defog, anti-flicker, lens correction, mirror, flip, gain, etc. |
| Audio | Audio Interface | 1-ch input (mic/pickup); 1-ch output (headphone/amplifier) |
|  | Audio Compression | G.711U / G.711A / AAC-LC |
| Network Connection | Simultaneous Preview | Up to 12 channels |
|  | Network Protocols | CGI, HTTP, HTTPS, GB28181, SIP, RTMP, TCP/IP, UDP, RTP, FTP, RTCP, RTSP, NTP, DHCP, DNS, DDNS, SNMP, SSL/TLS, WebSocket; ONVIF compliant |
|  | Browser | Plugin preview: IE10, IE11 Plugin-free preview: Edge, Chrome, Firefox |
| Auxiliary Interfaces | Network | 1× RJ45 10M/100M Ethernet port |
|  | Audio | 1-ch input (mic/pickup); 1-ch output (headphone/amplifier) |
|  | Light Board Ctrl | Light sensor detection input, PWM dimming supported |
|  | USB Interface | 1× USB 2.0 (5 V power); supports 4G / WiFi (dual-band) / storage / UVC expansion |
| Extension | Analog video | HD LVDS, CVBS for extension |
| General Parameters | Dimensions | 50× 50 × 100 mm |
|  | Weight | 100g |
|  | Operating Temp. | -20°C ~ +60°C |
|  | Power | DC 12 V input, 4 W power consumption |
| Lens & Model | Series | 4mm fixed Lens/F1.0 |
|  |  | 10x Optical Zoom Lens |
|  |  | 25x Optical Zoom Lens |
|  |  | 35x Optical Zoom Lens |